# CSW + Cisco Secure Firewall Integration Guide

NetFlow (NSEL) visibility and FMC policy enforcement — customer reference

Cisco Secure Workload User Education

2026-06-04

Table of Contents

# Cisco Secure Workload + Cisco Secure Firewall

NetFlow (NSEL) ingestion and policy enforcement on the firewall

Disclaimer: Companion learning material — not official Cisco product documentation. Validate design, supported versions, and limits against your tenant, release notes, and:

- FMC Integration Guide (Cisco.com)

- Secure Workload & Firewall deep dive (secure.cisco.com)

Architecture diagrams: ARCHITECTURE.md only — do not duplicate diagrams in other docs.

Scope / query / ACP mapping: SCOPE-ACP-QUERY-MAPPING.md — scope queries, inventory filters, WorkloadObj__ dynamic objects, Jason Maynard + Jorge Quintero video paths.

## Jorge Quintero — 3-part video series

Cisco Technical Marketing Engineer Jorge Quintero (Cisco Secure Workload YouTube channel) recorded the primary integration walkthrough. Watch before implementing.

| Part | Title | Topics covered | Watch |
| --- | --- | --- | --- |
| 1 | Secure Workload & Firewall Integration | Introduction, design, high-level architecture, use cases | YouTube |
| 2 | Secure Workload & Firewall Integration | Deployment patterns, NSEL ingest, FMC connector setup, policy flow | YouTube |
| 3 | Secure Workload & Firewall Integration | Enforcement, telemetry validation, operations, troubleshooting | YouTube |

Follow-up: Secure Workload & Secure Firewall Integration Updates (2025–2026) — latest behavior on the same channel.

Related Cisco Live session: Jorge Quintero — Solving the Segmentation Puzzle with Secure Workload (BRKSEC-2161).

## Executive summary

In hybrid multi-cloud environments, policy controls often fragment across host firewalls, network firewalls, SDN, and cloud security groups — managed by different teams. Cisco Secure Workload (CSW) and Cisco Secure Firewall together provide a unified microsegmentation path for workloads where host agents cannot be installed, while still supporting mixed agent + agentless estates.

Three integrated value streams:

| Use case | What it does | Primary path |
| --- | --- | --- |
| Microsegmentation | Discover agentless flows (NSEL), model policy (ADM), enforce on FTD via FMC | Secure Firewall Connector + FMC Connector |
| Virtual Patch | Export workload CVEs to FMC; apply compensating IPS rules until patches ship | FMC Connector (Virtual Patch tab) + agents |
| Rapid Threat Containment | FMC detects anomaly → Remediation Module → CSW quarantine guardrails | FMC Remediation Module + CSW labels |

## Solution overview

CSW delivers three platform capabilities that intersect with Secure Firewall:

- Zero Trust Microsegmentation — agent and agentless discovery, ADM-based policy, enforcement on host firewalls, DPUs, network firewalls, load balancers, and cloud controls.

- Vulnerability Detection and Protection — runtime CVE visibility from agents; virtual patch via Secure Firewall IPS.

- Behavioral Detection and Protection — process/forensic analytics; Rapid Threat Containment with FMC correlation + CSW guardrails.

## Architecture

See ARCHITECTURE.md for all customer-facing diagrams (high-level integration, NSEL/FMC sequences, scope mapping, insertion options, virtual patch, rapid threat containment). This guide covers steps and design detail only — no duplicate diagrams here.

### Two connectors — do not mix them

| Integration | Connector | Purpose | Data direction |
| --- | --- | --- | --- |
| Flow visibility (NetFlow / NSEL) | Secure Firewall Connector on Ingest appliance | CSW sees flows from FTD/ASA without agents | Firewall → Ingest → CSW |
| Policy enforcement | FMC Connector (Cisco Secure Firewall in UI) | CSW pushes segmentation to FTD via FMC | CSW → FMC → FTD |

| Also available | Use when |
| --- | --- |
| NetFlow connector | Switches/routers export NetFlow v9/IPFIX — not FTD NSEL |
| Host agents | Process-level visibility + host enforcement |
| ACI connector | Fabric policy — separate from this FTD/FMC guide |

## Workload protection level (design first)

Before choosing firewall insertion mode, define the security/trust boundary your organization uses (subnet, zone, app tier, persona). The white paper recommends aligning network engineers and app owners on a measurable protection level — commonly subnet-level for agentless FTD insertion.

| Persona | Typical boundary | Agent vs agentless |
| --- | --- | --- |
| App / DevOps | App tier, namespace | Often agent-based |
| Network / Firewall | Subnet, zone, VRF | Often agentless (this guide) |
| Cloud | VPC/VNet, security group | Mixed |

This drives whether you use L2 transparent (fine-grained), L3 routed (faster zone segmentation), or cloud hub patterns — see Insertion options.

## Prerequisites checklist

### CSW + Ingest (visibility)

| # | Requirement | Notes |
| --- | --- | --- |
| 1 | CSW tenant (SaaS or on-prem) with admin access | Scopes and workspaces ready |
| 2 | Secure Workload Ingest virtual appliance | Active before enabling connector |
| 3 | Secure Connector (SaaS) or network path | Mandatory for SaaS |
| 4 | FTD/ASA can reach Ingest on UDP 4729 | Copy IP:port from connector details |
| 5 | Capacity | 45k fps per connector; 135k fps per Ingest appliance (on-prem) |

### FMC (enforcement)

| # | Requirement | Notes |
| --- | --- | --- |
| 1 | FMC or cdFMC with supported FTD devices | FTD managed by FMC only |
| 2 | FMC REST API user with administrative privileges | Required for connector |
| 3 | FTD associated with FMC; ACP assigned; deploy verified | Test FMC→FTD deploy before CSW |
| 4 | Secure Connector if SaaS CSW cannot reach FMC directly | HTTPS 443 to FMC |
| 5 | FMC HA: include standby FMC IP in connector | Auto-switch on failover |
| 6 | Supported versions | Segmentation: FMC 7.0.x+ (dynamic objects 7.0.1+); Virtual Patch: 7.2+ |

### Virtual Patch (optional)

- CSW agents on target workloads

- Threat intelligence CVE data pack on CSW cluster

- Workloads use FTD as default gateway (traffic through IPS path)

- FMC network discovery sees hosts under Analysis → Network Map

## Part A — NetFlow / NSEL visibility (Secure Firewall Connector)

Goal: CSW receives bidirectional, stateful flow records from the firewall without host agents.

### Step A1 — Plan ingest placement

- Manage → Workloads → Virtual Appliances — confirm Ingest is Active.

- Record connector slot IP and port (4729 default).

- Open firewall path: UDP 4729 from FTD/ASA to Ingest.

- VRF: Manage → Agents → Configuration → Agent Remote VRF Configurations — one connector per VRF.

Videos: Connector Overview · Connector Deployment

### Step A2 — Enable Secure Firewall Connector in CSW

- Manage → Workloads → Connectors.

- Select Cisco Secure Firewall Connector (formerly ASA connector).

- Enable on Ingest appliance (not Edge-only).

- Record connector ID and IP:port from connector details page.

Limits: 1 Secure Firewall connector per Ingest appliance; 10 per tenant.

### Step A3 — Enable NSEL on Secure Firewall

ASA example (replace <INGEST_CONNECTOR_IP>):

flow-export destination outside <INGEST_CONNECTOR_IP> 4729
flow-export template timeout-rate 1
!
policy-map flow_export_policy
  class class-default
  flow-export event-type flow-create destination <INGEST_CONNECTOR_IP>
  flow-export event-type flow-teardown destination <INGEST_CONNECTOR_IP>
  flow-export event-type flow-denied destination <INGEST_CONNECTOR_IP>
  flow-export event-type flow-update destination <INGEST_CONNECTOR_IP>
  user-statistics accounting
service-policy flow_export_policy global

FTD: Configure NSEL export via FMC/device template — ASA NetFlow Implementation Guide.

Videos (Jorge Quintero, 3-part series): Part 1 · Part 2 · Part 3 · 2025–2026 updates

### Step A4 — Validate flow ingestion

- Generate permitted and denied test traffic.

- Flow Analysis — confirm firewall-sourced endpoints.

- Connector heartbeat healthy under Manage → Workloads → Connectors.

- Optional API filters: user_src_NETFLOW_IDENTIFIED / user_dst_NETFLOW_IDENTIFIED.

NSEL handling: flow-create/update → exported; flow-denied → rejected disposition; connector sends forward + reverse flows; NAT flow-stitching for end-to-end visibility.

Video: Flow Analysis

### Step A5 — Label workloads and run discovery

- Import labels from CMDB (ServiceNow), IPAM, or manual assignment.

- Run ADM on scoped workspace.

- Remain in Monitor mode until app owners validate dependencies.

Videos: Basic Application Discovery · Enhancing Application Discovery · ADM & Policy Analysis

## Part B — Policy enforcement on Secure Firewall (FMC Connector)

Goal: CSW pushes L3/L4 segmentation to FTD via FMC using Dynamic Objects — inventory changes refresh membership without manual object edits.

CSW orchestrated rules are L3/L4 east-west microsegmentation. Layer-7 FMC features should not be added to CSW-controlled rules (not supported).

### Step B1 — Onboard firewalls in FMC

- FTD devices registered to FMC or cdFMC.

- Each FTD assigned to an Access Control Policy (ACP).

- Verify FMC can deploy to FTD and traffic behaves as expected.

- Document one scope → one ACP mapping plan (1:1 only).

Video: FMC Integration (Edge / Ingest / Appliance)

### Step B2 — Create FMC Connector in CSW

- Manage → Workloads → Connectors → Firewall → Cisco Secure Firewall.

- Click Configure your new connector here.

- Complete New Connection fields:

| Field | Description |
| --- | --- |
| Connector Name | Unique name for this FMC connection |
| Username / Password | FMC REST API credentials (admin privileges) |
| CA Certificate | FMC CA cert for TLS validation, or Disable SSL on trusted networks |
| Server IP/FQDN and Port | FMC hostname or IP (443) |
| HTTP Proxy | If required: <proxy.host>:<proxy.port> |
| Secure Connector | Enable when SaaS CSW reaches FMC through Secure Connector |

- Click Create; verify Event Log tab for errors.

Official procedure: FMC Integration Guide — Connection Settings

### Step B3 — Map scope to Access Control Policy

Prerequisite: Scope queries committed and inventory filters validated — SCOPE-ACP-QUERY-MAPPING.md.

- Open FMC connector → Segmentation tab → + Add.

- Add ACP Mapping:

  - Select Access Policy (ACP) from FMC.

  - Map to exactly one Scope.

  - Use Secure Workload Catch-All — CSW catch-all in Default section, or use FMC default action.

  - Enforcement Mode:

    - Merge — CSW rules coexist with FMC rules; configure priority.

    - Override — CSW replaces user-created rules in mapped sections.

  - Absolute / Default Policies priority (Merge only):

    - Insert above or below existing Mandatory/Default rules per section.

- Click Submit.

#### Scope mapping strategy

| Model | When | Behavior |
| --- | --- | --- |
| Child / leaf scope | Single application behind firewall | Push that app's policies + parent guardrails |
| Parent scope | Multiple applications | Push parent + eligible child policies (hierarchical) |

#### Topology awareness

CSW pushes rules only to FTD devices on the traffic path for the mapped scope — avoids rule sprawl across unrelated firewalls.

Videos: Policy Enforcement Overview · Where to Enforce · Policy Ordering

### Step B4 — What CSW creates in FMC

When enforcement is enabled on a workspace, CSW converts policies to FMC rules:

| FMC artifact | Prefix | ACP section | Purpose |
| --- | --- | --- | --- |
| Golden rules | Workload_golden__ | Mandatory | CSW ↔ agent communication behind FW |
| Segmentation rules | Workload__ | Mandatory / Default | Enforced workspace policies |
| Catch-all rules | Workload_ca__ | Default | If Use Secure Workload Catch-All enabled |
| Dynamic objects | WorkloadObj__ | Objects | Scope/inventory IP membership |

| CSW policy type | FMC placement |
| --- | --- |
| Absolute | Mandatory category |
| Default | Default category |

Do not manually edit Workload_* rules in FMC — CSW restores them on next push. Independent FMC rules are safe in Merge mode if they avoid these prefixes.

### Step B5 — Model policy, then enforce

- Build or accept ADM-recommended policies in workspace tied to scope.

- Simulate / Monitor — Policy Visual and flow analysis.

- Change window: enable enforcement on workspace.

- Confirm FMC deployment succeeds; verify Dynamic Objects update as inventory changes.

Videos: Policy Lifecycle · Policy Validation and Analysis

### Step B6 — Validate enforcement

| Test | Expected result |
| --- | --- |
| Allowed business flow | Passes; CSW/FMC show permit |
| Intentionally denied flow | Blocked at FTD; CSW rejected-flow evidence |
| New workload in scope | WorkloadObj__ membership updates |
| FMC deployment | Automatic on CSW policy change |
| Compliance monitoring | Alerts on rejected flows / policy deviation |

Check Event Log on FMC connector for push errors (Information / Warning / Error).

## Part C — Virtual Patch (optional)

Compensating IPS control for CVEs until permanent patches are deployed.

- FMC connector → Virtual Patch tab → Create a Virtual Patching Rule.

- Define Host Query (scope/filter) and CVE Query (CVSS, Cisco Risk Score, etc.).

- CSW exports matching CVEs to FMC Third-Party Vulnerabilities.

- FMC admin runs Cisco Recommended Rules to map CVEs → Snort signatures.

- Apply IPS policy on ACP rule for affected traffic flows.

Prerequisites: Agents on workloads; CVE data pack; FTD as gateway; FMC 7.2+.

## Part D — Rapid Threat Containment (optional)

Automated quarantine when FMC detects malicious behavior:

- Install FMC Remediation Module for Secure Workload on FMC.

- Configure CSW cluster IP, root scope, API key (User Data Upload permission).

- Define Correlation Rules and Correlation Policy responses.

- On trigger, Remediation Module sends affected IPs to CSW via API.

- CSW guardrail policies (quarantine labels) enforce across agents and FTD.

## Firewall insertion options

| Mode | Segmentation | NSEL visibility | Best fit |
| --- | --- | --- | --- |
| L2 Transparent | Intra- + inter-subnet | Full | Legacy OS, fine-grained, localized |
| L3 Routed | Inter-subnet only | Inter-subnet only | Quick zone segment, dev/non-prod |
| ACI Service Graph PBR | Intra + inter EPG/ESG | Full (FW in path) | ACI + FTD multi-policy |
| AWS hub VPC | Inter-VPC | VPC logs + NSEL | Centralized cloud east-west |
| AWS distributed | Intra-VPC | VPC logs + NSEL | Per-VPC enforcement |
| Azure hub VNet | Intra- + inter-VNet | NSG logs + NSEL | UDR steering |
| GCP hub VPC | Inter-VPC | VPC logs + NSEL | Centralized GCP |

All patterns support dual management: CSW east-west + FMC north-south on the same ACP (Merge mode).

## FAQs (from Cisco white paper)

| Question | Answer |
| --- | --- |
| Mixed agent + agentless behind same mapped firewall? | Works. Agent-based rules may also appear in ACP (hierarchical policy). Primary use case remains agentless protection. |
| Map multiple scopes to one ACP? | No — strictly 1 scope : 1 ACP. |
| Add Layer-7 / FMC features to CSW rules? | Not supported — CSW rules are L3/L4 east-west only. |
| Dual-management (CSW + FMC rules)? | Merge mode + rule ordering; preserve order after go-live. |
| Modify a CSW-owned rule in FMC? | CSW overwrites on next push. |
| Supported FMC versions? | Dynamic objects: 7.0.1+; Virtual Patch: 7.2+; qualified: 7.0.1, 7.2.5. |

## Recommended video learning path

| Order | Video | Presenter / channel | Link |
| --- | --- | --- | --- |
| 1 | Secure Workload & Firewall Integration — Part 1 | Jorge Quintero · @ciscosecureworkload | Watch |
| 2 | Part 2 — deployment & policy flow | Jorge Quintero | Watch |
| 3 | Part 3 — enforcement & operations | Jorge Quintero | Watch |
| 4 | Integration Updates (2025–2026) | Cisco Secure Workload channel | Watch |
| 5 | Connector Overview | Cisco Secure Workload channel | Watch |
| 6 | Connector Deployment | Cisco Secure Workload channel | Watch |
| 7 | FMC + Edge / Ingest / Appliance | Cisco Secure Workload channel | Watch |
| 8 | Where to Enforce | Cisco Secure Workload channel | Watch |
| 9 | Policy Enforcement Overview | Cisco Secure Workload channel | Watch |

## NetFlow connector vs Secure Firewall connector vs API flowsearch

| Method | Use when | Scale notes |
| --- | --- | --- |
| Secure Firewall Connector (NSEL) | FTD/ASA agentless visibility | 45k fps per connector; 135k fps per Ingest |
| NetFlow connector | Switches/routers (v9/IPFIX) | ~15k fps per connector |
| Host agents | Process-level + host enforce | Best fidelity |
| OpenAPI flowsearch | Query ingested flows | Paginate; ≤24h; 1h chunks — not bulk ingest |

## POV evidence checklist

| Evidence | Proves |
| --- | --- |
| Connector enabled + heartbeat healthy | Ingest path live |
| Sample flows in Flow Analysis (NSEL) | Visibility |
| ADM map for scoped app | Discovery value |
| FMC connector connected + Event Log clean | Enforcement path |
| Scope ↔ ACP mapping screenshot | Topology binding |
| Workload__ / WorkloadObj__ in FMC | Policy push working |
| Monitor-mode policy stats | Safe pre-enforce review |
| Post-enforce deny test + FMC hit | Enforcement works |
| Allowed transaction test | No business break |

## Common pitfalls

| Pitfall | Fix |
| --- | --- |
| Confusing NetFlow connector with Secure Firewall connector | NSEL from FTD/ASA → Secure Firewall Connector only |
| Expecting NSEL alone to enforce | Requires FMC connector + workspace enforce |
| Editing Workload_* rules in FMC | CSW will overwrite — change policy in CSW |
| Multiple scopes on one ACP | Redesign — 1:1 mapping only |
| L3 routed FW for intra-subnet segment | Use L2 transparent or host agents |
| Bulk flowsearch API timeouts | Paginate; chunk time windows |
| Skipping Secure Connector on SaaS | Required for Ingest and FMC reachability |

## Official Cisco documentation

| Document | URL |
| --- | --- |
| FMC Integration Guide | cisco-secure-workload-and-fmc-integration-guide.html |
| Deep dive white paper | secure-workload-whitepaper |
| CSW 4.0 SaaS — Connectors | m-connectors.html |
| Marketing white paper | sec-workload-firewall-wp.html |
| ASA NSEL configuration | ASA NetFlow Implementation Guide |

## Related repos

- CSW-Secure-Firewall-Integration-Guide — this repo

- CSW-User-Education — video library and onboarding

- CSW-Policy-Lifecycle — ADM → Monitor → Enforce

- CSW-Agent-Installation-Guide — host agents

- CSW-Identity-Integration-Guide — AD / Entra identity

- CSW-ServiceNow-Connector-Guide — CMDB labels

- csw-splunk-integration — SIEM

- CSW-Compliance-Mapping — compliance frameworks

- csw-logs-check — host enforce timing from agent logs

# CSW + Secure Firewall — Architecture Reference

Single source of truth for all architecture diagrams in this repo. Other documents link here instead of repeating diagrams.

Video walkthrough: Cisco TME Jorge Quintero — 3-part series on @ciscosecureworkload: Part 1 — design · Part 2 — deployment · Part 3 — operations

Derived from the Cisco Secure Workload & Secure Firewall deep dive and the FMC Integration Guide.

Disclaimer: Companion material — validate against your CSW and FMC release notes before production.

## 1. High-level integration (microsegmentation)

This is the reference architecture for agentless east-west segmentation using NSEL visibility and FMC enforcement.

flowchart TB
  subgraph Sources["Context & telemetry"]
    CMDB["CMDB / ServiceNow"]
    IPAM["IPAM / DNS"]
    Labels["Manual labels"]
  end

  subgraph Workloads["Application workloads"]
    Agent["Agent-based workloads<br/>host firewall enforce"]
    Agentless["Agentless workloads<br/>legacy / appliance / no agent"]
  end

  subgraph Visibility["Visibility path — NSEL"]
    FTD["Secure Firewall FTD / ASA<br/>Transparent or Routed mode"]
    NSEL["NSEL export<br/>NetFlow v9 · UDP 4729"]
    Ingest["Secure Workload Ingest Appliance"]
    SFC["Secure Firewall Connector<br/>Docker · up to 45k fps each"]
    FTD -->|"flow-create · teardown · deny · update"| NSEL --> SFC
    SFC --> Ingest
  end

  subgraph CSW["Cisco Secure Workload"]
    FlowDB["Flow analysis & inventory"]
    ADM["Application Dependency Mapping"]
    Policy["Policy engine<br/>Monitor → Simulate → Enforce"]
    Ingest --> FlowDB
    CMDB --> FlowDB
    IPAM --> FlowDB
    Labels --> FlowDB
    FlowDB --> ADM --> Policy
  end

  subgraph Enforcement["Enforcement path — FMC"]
    FMCConn["FMC Connector<br/>HTTPS REST API :443"]
    FMC["Secure Firewall Management Center"]
    ACP["Access Control Policy<br/>Dynamic Objects"]
    Policy -->|"Scope ↔ ACP mapping<br/>Topology-aware rules"| FMCConn
    FMCConn --> FMC --> ACP
    ACP -->|"auto-deploy"| FTD
  end

  Agentless -.->|"traffic through FW"| FTD
  Agent -.->|"optional: rules also pushed to ACP"| Policy

Key idea: Visibility and enforcement use different connectors. NSEL tells CSW what flows exist; the FMC connector pushes L3/L4 segmentation policy to FTD when you enforce a workspace.

## 2. Data flow — visibility (NSEL)

sequenceDiagram
  participant W as Workload traffic
  participant FW as Secure Firewall FTD/ASA
  participant IC as Secure Firewall Connector
  participant CSW as Secure Workload

  W->>FW: Application flow
  FW->>FW: Stateful inspection
  FW->>IC: NSEL record (create/update/deny/teardown)
  Note over IC: Decapsulate NetFlow v9<br/>Forward + reverse flows<br/>NAT flow-stitching
  IC->>CSW: Flow observations (agentless)
  CSW->>CSW: ADM · labels · policy modeling

| NSEL capability | Customer value |
| --- | --- |
| Stateful events | See permits and denies — not just sampled NetFlow |
| Bidirectional flows | Connector exports forward + reverse observations |
| Flow stitching | End-to-end visibility when NAT is in path |
| Agentless discovery | No agent on legacy OS, appliances, or restricted hosts |

Scale (on-prem Ingest, per Cisco white paper):

| Metric | Limit |
| --- | --- |
| Per Secure Firewall connector | Up to 45,000 fps |
| Per Ingest appliance (total) | Up to 135,000 fps |
| Connectors per Ingest appliance | 1 Secure Firewall connector |
| Connectors per tenant | 10 |

## 3. Data flow — enforcement (FMC)

sequenceDiagram
  participant Op as Operator
  participant CSW as Secure Workload
  participant FMC as FMC / cdFMC
  participant FTD as FTD cluster

  Op->>CSW: Enable enforcement on workspace
  CSW->>CSW: Resolve scope inventory → IP sets
  CSW->>FMC: Create/update Dynamic Objects (WorkloadObj_*)
  CSW->>FMC: Push ACP rules (Workload_* · Workload_golden_* · Workload_ca_*)
  FMC->>FTD: Deploy policy (no manual redeploy for CSW changes)
  Note over FTD: Topology awareness — rules only on firewalls in traffic path

### FMC rule prefixes (do not edit in FMC)

| Prefix | Purpose | ACP section |
| --- | --- | --- |
| Workload_golden__ | Allow CSW ↔ agents behind the firewall | Mandatory |
| Workload__ | Segmentation rules from enforced workspaces | Mandatory / Default |
| Workload_ca__ | Catch-all rules (if Use Secure Workload Catch-All enabled) | Default |
| WorkloadObj__ | Dynamic objects for scope/inventory membership | Objects |

CSW overwrites changes to rules using these prefixes on the next push. Independent FMC rules are preserved in Merge mode if they do not use these prefixes.

## 4. Scope ↔ ACP mapping models

flowchart LR
  subgraph Single["Single application"]
    CS1["Child / leaf scope"]
    ACP1["One ACP"]
    CS1 -->|"1:1 mapping"| ACP1
  end

  subgraph Multi["Multiple applications"]
    Parent["Parent scope"]
    C1["Child scope A"]
    C2["Child scope B"]
    ACP2["One ACP"]
    Parent --> C1
    Parent --> C2
    Parent -->|"hierarchical policy"| ACP2
  end

| Mapping | When to use | What CSW pushes |
| --- | --- | --- |
| Child scope → ACP | One application protected by this firewall pair | Policies for that app + parent guardrails |
| Parent scope → ACP | Multiple apps behind same firewall estate | Parent + eligible child scope policies |

Constraint: One ACP maps to one scope only (1:1). Plan ACP boundaries accordingly.

## 5. ACP mapping options

| Setting | Merge mode | Override mode |
| --- | --- | --- |
| Existing FMC rules | Kept; CSW rules inserted top or bottom | Replaced by CSW rules |
| Rule priority | Configurable: above/below Mandatory and Default sections | N/A |
| Catch-all | Optional: CSW catch-all or FMC default action | Same |
| Dual management | Yes — network team keeps north-south / baseline rules | CSW-only in mapped sections |

Policy type placement:

| CSW policy type | FMC ACP section |
| --- | --- |
| Absolute policies | Mandatory category |
| Default policies | Default category |

## 6. Firewall insertion options (on-premises)

Choose insertion based on protection level (typically subnet / zone boundary) and segmentation granularity.

flowchart TB
  subgraph L2["Layer 2 — Transparent / bump-in-wire"]
    L2W["Fine-grained · intra + inter-subnet<br/>Full NSEL visibility<br/>Legacy OS / localized workloads"]
  end

  subgraph L3["Layer 3 — Routed / gateway"]
    L3W["Faster time-to-segment · inter-subnet only<br/>Partial NSEL (no intra-subnet)<br/>Dev / non-prod zones"]
  end

  subgraph ACI["ACI + Service Graph"]
    ACIW["Intra + inter EPG/ESG<br/>PBR redirect or go-to/through<br/>CSW + FMC + ACI policies"]
  end

| Insertion | Segmentation | NSEL visibility | Best for |
| --- | --- | --- | --- |
| L2 Transparent | Intra- + inter-subnet | Full (intra + inter) | Fine-grained, legacy OS, localized apps |
| L3 Routed | Inter-subnet only | Inter-subnet only | Quick zone segmentation, distributed workloads |
| ACI Service Graph (PBR) | Intra + inter EPG/ESG | Full with FW in path | ACI fabric + FTD service graph |
| ACI Go-To / Go-Through | Inter or intra+inter | Varies by mode | North-south or strict in-path designs |

## 7. Cloud insertion options (summary)

| Platform | Pattern | East-west protection | Visibility |
| --- | --- | --- | --- |
| AWS | Centralized hub VPC FTD | Inter-VPC / inter-subnet | VPC flow logs + NSEL |
| AWS | Distributed per-VPC FTD | Intra-VPC inter-subnet | VPC flow logs + NSEL |
| Azure | Hub VNet FTD + UDR | Intra- + inter-VNet | NSG flow logs + NSEL |
| GCP | Hub VPC FTD | Inter-VPC | VPC flow logs + NSEL |

All cloud patterns support FMC dual management: CSW owns east-west microsegmentation; FMC owns north-south ingress/egress.

## 8. Extended use cases

### Virtual Patch (agent + FMC)

flowchart LR
  Agent["CSW agent on workload"] -->|"CVE inventory"| CSW
  CSW -->|"FMC Connector Virtual Patch tab"| FMC
  FMC -->|"Third-Party Vulnerabilities + Snort mapping"| IPS["IPS policy / Cisco Recommended Rules"]
  IPS --> FTD

- Requires agents on workloads where virtual patch applies.

- Workloads must use an FTD address as default gateway for traffic to hit IPS.

- FMC 7.2+ for virtual patch; CSW publishes CVEs → FMC → fine-tuned IPS rules.

### Rapid Threat Containment

flowchart LR
  Anomaly["Malware / IPS / correlation event"] --> FMC
  FMC -->|"Remediation Module API"| CSW
  CSW -->|"Quarantine label / guardrail policy"| Enforce["Host agent + FTD enforce"]

- FMC Remediation Module sends affected IPs to CSW.

- CSW guardrail policies (label-based) quarantine workloads across enforcement points.

## 9. Connectivity requirements

| Path | Protocol | Port | Notes |
| --- | --- | --- | --- |
| FTD/ASA → Ingest | UDP (NSEL) | 4729 (default) | Firewall must reach Ingest slot IP |
| CSW → FMC | HTTPS | 443 | REST API; admin credentials |
| SaaS CSW ↔ on-prem | Secure Connector | tunneled | Required for SaaS + on-prem FMC/Ingest |
| FMC HA | Include standby FMC IP | 443 | Auto-failover to new active FMC |

## 10. Supported versions (reference)

| Capability | CSW | FMC / FTD |
| --- | --- | --- |
| Segmentation (ACP mapping, topology) | 3.9.1.1+ | 7.0.x+ (dynamic objects: 7.0.1+) |
| Virtual Patch | 3.8.1.1+ | 7.2+ |
| Simplified segmentation workflow | 3.9.1.1+ | 7.2 recommended |
| Engineering qualified (white paper) | — | 7.0.1, 7.2.5 |

Platforms: FTD devices managed by FMC only (not standalone ASA for enforcement path). FTD Transparent and Routed modes supported.

## Official references

| Document | URL |
| --- | --- |
| Deep dive white paper | secure.cisco.com/secure-workload/docs/secure-workload-whitepaper |
| FMC integration guide | cisco-secure-workload-and-fmc-integration-guide.html |
| Marketing white paper | sec-workload-firewall-wp.html |

# Scope, Query, Inventory Filters & ACP Mapping

How CSW scope queries, inventory filters, and clusters become FMC Dynamic Objects (WorkloadObj__*) and ACP rules — with video and documentation references.

Disclaimer: Companion material. Authoritative sources:

- FMC Integration Guide

- Secure Workload & Firewall deep dive

- CSW User Guide — Scopes, Inventory Filters, Manage Policy Lifecycle

Architecture diagrams: ARCHITECTURE.md · Firewall integration steps: INTEGRATION-GUIDE.md

## Terminology — do not confuse these

| Concept | What it is | Role in firewall enforcement |
| --- | --- | --- |
| Scope | Hierarchical container for policy (parent/child tree) | One scope maps to one FMC ACP (1:1) |
| Scope query | Boolean query defining which workloads belong in a scope | Drives scope membership; changes need commit + impact review |
| Label | Key/value metadata on workloads (CMDB, cloud tag, manual) | Used inside scope queries and policy consumers/providers |
| Inventory filter | Named, reusable query (scope-bound) for grouping workloads | Becomes IP sets → WorkloadObj__* dynamic objects in FMC |
| Cluster | ADM-derived workload group (often ephemeral) | Can be converted to inventory filter for stable enforcement |
| WorkloadObj__* | FMC Dynamic Object created by CSW FMC connector | Membership updates automatically when inventory changes |
| Workload__* / Workload_ca__* | FMC ACP rules pushed from enforced CSW workspaces | L3/L4 segmentation rules referencing WorkloadObj__ objects |
| CSDAC (FMC Dynamic Attribute Connector) | Separate FMC feature for cloud tag → FMC objects | AWS/Azure/GCP tags — not the CSW WorkloadObj__ path |

Key FMC integration quote (Cisco docs):

Secure Workload enforced segmentation policies are converted into Access Control Policies, utilizing IP address sets derived from scopes, inventory filters, and clusters. These sets are transformed into dynamic objects within FMC.

Network inventory is dynamically updated from inventory filters; when workloads are added, changed, or removed, CSW updates FMC Dynamic Objects and auto-deploys to FTD — no manual FMC redeploy for object membership changes.

## End-to-end mapping chain

flowchart LR
  subgraph CSW["Cisco Secure Workload"]
    L["Labels<br/>CMDB · cloud · manual"]
    SQ["Scope query"]
    IF["Inventory filters<br/>+ clusters"]
    POL["Policies<br/>consumer / provider"]
    WS["Workspace<br/>Monitor → Enforce"]
    L --> SQ
    SQ --> IF
    IF --> POL
    POL --> WS
  end

  subgraph FMC["FMC / FTD"]
    MAP["ACP ↔ Scope mapping<br/>1:1 · topology-aware"]
    DO["Dynamic Objects<br/>WorkloadObj__*"]
    RULES["ACP rules<br/>Workload__* · Workload_golden__*"]
    WS -->|"FMC Connector"| MAP
    IF --> DO
    POL --> RULES
    DO --> RULES
    RULES --> FTD["FTD deploy"]
  end

Order of operations: Labels → Scope tree + queries → Inventory filters → ADM/policy → then FMC connector ACP mapping → enforce workspace.

## Phase 0 — Foundations (Jason Maynard)

Jason Maynard (How Hard Can It Be? series) covers CSW building blocks on his YouTube channel. Complete before ACP mapping — Jorge Quintero's firewall series assumes you understand scopes, labels, and filters.

| Order | Video | Steps / topics (from catalog) | Watch |
| --- | --- | --- | --- |
| 1 | Cisco Secure Workload: Scopes | Create scope tree; group workloads for policy; parent/child hierarchy | YouTube |
| 2 | Cisco Secure Workload: Labels | Tag workloads; use labels in queries and policy | YouTube |
| 3 | Cisco Secure Workload: Inventory Filters | Build filters to focus inventory; reusable query groups | YouTube |
| 4 | Application Dependency Mapping & Policy Analysis | Map dependencies; derive segmentation policy from flows | YouTube |
| 5 | Dynamic Workloads & Policy | Policy adapts as workloads move or scale | YouTube |
| 6 | Policy Visual and Quick Analysis | Visualize policy impact before enforce | YouTube |
| 7 | Production and Test Risk Reduction | Macro-segment prod vs non-prod (common first firewall scope) | YouTube |
| 8 | Flow Analysis | Validate NSEL/agent flows before policy (pairs with Part A of firewall guide) | YouTube |

Official channel refresh: Inventory Filters (2025–2026) — @ciscosecureworkload

## Phase 1 — Scope query (CSW UI / API)

Define which workloads belong in each scope before FMC mapping.

### Steps (CSW User Guide + OpenAPI)

| Step | Action | Notes |
| --- | --- | --- |
| 1 | Manage → Scopes (or tenant scope tree) | Plan parent (multi-app) vs child/leaf (single app) before FMC |
| 2 | Create a New Scope | Name aligned to app or zone boundary (e.g. Prod-Payments-App) |
| 3 | Edit scope query | Use labels, IP, hostname, agent attributes, connector metadata |
| 4 | Review Scope Query Change Impact | UI modal shows affected workloads and policies |
| 5 | Commit scope query changes | API: POST /scopes/{id}/commit_query_changes — until committed, query is draft |
| 6 | View workloads in scope | Confirm count matches expectation before enforce |
| 7 | Policy inheritance | Child scopes inherit parent guardrail policies (relevant for leaf → ACP mapping) |

### Scope mapping strategy for FMC

| FMC mapping | Map this scope type | CSW pushes |
| --- | --- | --- |
| Single application | Child / leaf scope | That app's policies + parent guardrails |
| Multiple applications | Parent scope | Parent + eligible child scope policies (hierarchical) |

Constraint: One ACP ↔ one scope only. Plan one FMC Access Control Policy per segmentation boundary.

## Phase 2 — Inventory filters & clusters

Inventory filters are the primary query objects that become WorkloadObj__* IP sets in FMC.

### Steps (CSW User Guide)

| Step | Action | Notes |
| --- | --- | --- |
| 1 | Create an Inventory Filter | Scope-bound; name reflects consumer/provider role (e.g. web-tier, db-tier) |
| 2 | Validate an inventory filter query | API: validate before save — check match count |
| 3 | Review Filter Change Impact | Shows policy/workspace impact before commit |
| 4 | Run ADM / accept clusters | ADM may propose clusters as workload groups |
| 5 | Convert a Cluster to an Inventory Filter | Stabilize ADM output for enforcement |
| 6 | Use filters as policy consumer/provider | Policies reference filters, not raw IP lists |
| 7 | Dynamic Workloads & Policy | When labels/IPs change, filter membership refreshes → FMC objects update |

### Query building tips

- Combine labels (from ServiceNow, cloud connectors, manual) with IP/subnet for agentless NSEL-visible hosts.

- Keep filter names stable — FMC WorkloadObj__ names derive from CSW objects.

- Prefer inventory filters over ad-hoc IP lists in policies for FMC dynamic object hygiene.

## Phase 3 — Policy workspace (before FMC push)

| Step | Action | Notes |
| --- | --- | --- |
| 1 | Create workspace tied to target scope | Primary workspace for the app |
| 2 | ADM or manual policies — consumer/provider use inventory filters | See Jason Maynard ADM + Policy Visual videos |
| 3 | Monitor / Simulate | No FTD push yet |
| 4 | Policy Visual — allowed/denied impact | Validate with app owner |
| 5 | Document Absolute vs Default vs Catch-All rank | Maps to FMC Mandatory vs Default sections |

## Phase 4 — ACP ↔ Scope mapping (FMC Connector)

After scope queries, filters, and monitor-mode policy are validated — configure FMC connector Segmentation tab.

### Prerequisites

- FMC connector connected (Event Log clean)

- FTD devices assigned to target ACP; FMC deploy verified

- One scope selected per ACP

### Steps (FMC Integration Guide)

| Step | UI action | Detail |
| --- | --- | --- |
| 1 | Manage → Workloads → Connectors → Cisco Secure Firewall | Open existing FMC connection |
| 2 | Segmentation tab → + Add | Add ACP Mapping |
| 3 | Choose Access Policy (ACP) from FMC dropdown | Must match FTD assignment in FMC |
| 4 | Map to one Scope | 1:1 — cannot map multiple scopes to one ACP |
| 5 | Use Secure Workload Catch-All | CSW catch-all in Default section, or FMC default action |
| 6 | Enforcement Mode | Merge (dual-management) or Override (CSW-only in section) |
| 7 | Absolute / Default Policies priority | Merge only: insert above/below Mandatory/Default rules |
| 8 | Submit | Topology awareness pushes rules only to FTDs on traffic path |
| 9 | Enable enforcement on CSW workspace | Triggers push of objects + rules |
| 10 | Verify in FMC | Objects: WorkloadObj__* · Rules: Workload__*, Workload_golden__*, Workload_ca__* |
| 11 | Event Log tab | Filter errors (red); download JSON/CSV if needed |

### What CSW creates in FMC

| FMC artifact | Prefix | Section |
| --- | --- | --- |
| Golden rules (CSW ↔ agents behind FW) | Workload_golden__ | Mandatory |
| Segmentation rules from enforced workspace | Workload__ | Mandatory / Default |
| Catch-all (if enabled) | Workload_ca__ | Default |
| Dynamic objects from scopes/filters/clusters | WorkloadObj__ | Object Management |

| CSW policy rank | FMC ACP category |
| --- | --- |
| Absolute | Mandatory |
| Default | Default |

Do not edit Workload_* rules or WorkloadObj__ objects manually in FMC — CSW overwrites on next push.

### Edit / view mapping later

- Defend → Segmentation → select policy → pencil icon → Submit → Save

- Domains tab (FMC connector config) — view which FMC domains are enforced (3.9.1.1+)

## Phase 5 — Firewall integration videos (Jorge Quintero)

After Phase 0–4 concepts, watch Jorge's series for NSEL + FMC connector deployment:

| Part | Focus | Watch |
| --- | --- | --- |
| 1 | Design & architecture | YouTube |
| 2 | Deployment & policy flow | YouTube |
| 3 | Enforcement & operations | YouTube |

## CSW WorkloadObj__ vs FMC CSDAC (Dynamic Attributes)

|  | CSW → FMC (WorkloadObj__) | FMC CSDAC |
| --- | --- | --- |
| Source | CSW scopes, inventory filters, clusters | AWS/Azure/GCP/VMware tags via FMC connector |
| Configured in | CSW UI + FMC connector Segmentation | FMC → Dynamic Attributes Connector |
| Use with CSW firewall integration | Yes — required for CSW-driven segmentation | Optional — parallel cloud-native object path |
| Doc | FMC Integration Guide | CSDAC overview |

For a CSW POV with FMC enforcement, you need the CSW FMC connector path. CSDAC is complementary when FMC also needs cloud tag-based objects outside CSW policy.

## Validation checklist

| # | Check | Pass criteria |
| --- | --- | --- |
| 1 | Scope query committed | Workload count stable; impact review done |
| 2 | Inventory filters validated | Match expected tiers; names stable |
| 3 | Policies use filters (not static IPs) | Consumer/provider reference filters/clusters |
| 4 | Workspace in Monitor | ADM/policy validated with app owner |
| 5 | ACP mapping 1:1 | One scope per ACP; topology correct |
| 6 | FMC WorkloadObj__* populated | Objects show current IPs |
| 7 | Post-enforce deny test | Blocked at FTD; CSW shows rejected flow |
| 8 | Add/remove test workload | Filter membership + FMC object update without manual FMC edit |

## FAQs (scope / query / ACP)

| Question | Answer |
| --- | --- |
| Multiple scopes on one ACP? | No — strictly 1:1 |
| Mixed agent + agentless behind same FW? | Works; agent host rules may also appear in ACP (hierarchical policy) |
| Edit Workload__ rule in FMC? | CSW restores on next push |
| Layer 7 on CSW rules? | Not supported — L3/L4 east-west only |
| Scope query change after enforce? | Review impact → commit → FMC objects refresh on push |
| Cluster vs inventory filter? | Convert clusters to filters for stable FMC object mapping |

## Recommended learning order

Jason Maynard: Scopes → Labels → Inventory Filters → ADM → Policy Visual
    → Jorge Quintero: Firewall Integration Parts 1–3
    → This doc: Scope query → Filters → ACP mapping → Enforce
    → INTEGRATION-GUIDE.md: NSEL ingest + FMC connector setup detail